# Prepare GFF

Muffle furnace heat the 0.7 mm glass fiber filters (GF/F) at 400 C for 2 hours (small batches wrapped in aluminum foil). Make sure that the filters are not sitting directly on the coils to avoid damaging the filters. Avoid directly touching filters with fingers (use forceps or wear gloves).

# Sample processing with GFF

- Shake sample (invert ~5 times) to ensure solids are suspended

- Immediately after inverting, filter 200 mL through a 0.7 mm GF/F filter using vacuum filtration

  - Use forceps to place the filter on the apparatus to avoid touching with fingers. Wear gloves.

  - Triple rinse the filtration equipment with DI or ultrapure water between samples from different sites. Air dry filtration equipment as best as possible to avoid diluting the sample with DI.

- Transfer 100 mL of filtered water to 125 mL HDPE bottle to ship to Kyle (Gonzaga)

- Transfer 100 mL of filtered water to 125 mL HDPE bottle for Ryan (coordinate on getting samples back to Ryan)

- Transfer remaining unfiltered 800 mL to 1 L glass amber bottle (labeled) for Xue’s lab