Standard Operating Procedure for the Field Collection and Preparation of Dissolved Organic Carbon Concentration and Character Samples

Prepared by Katie Wampler

- Scope and Application

  - This method details the steps used to collect surface water samples from freshwater streams to use for determining dissolved organic carbon concentrations using the EPA Method 415.1 and dissolved organic carbon character using excitation emission matrices.

- Safety

  - Care should be taken when collecting water samples, particularly during high flows in larger streams. Appropriate safety gear including waders, life vest, hard hat/helmet, safety lines, and the buddy system should be used.

  - Always wear gloves when handling and collecting samples to prevent sample contamination.

- Equipment and Supplies

  - Safety

    - Hard Hat/Helmet

    - High Vis Vest

    - Waders

    - Life Vest

    - Rope for Safety Line

    - Spot Unit

  - Sample Collection

    - Acid washed, HPDE 125mL bottles (including ~10+ extra)

    - Nitrile Gloves

    - 60 mL plastic syringes

    - 0.2 um syringe filters

    - 0.45 um syringe filters

    - Labeling Tape

    - Sharpies

    - Pencils

    - Extendable Swing Sampler

    - Datasheets and Clipboard

    - Cooler

    - Icepacks

    - Tablet or Phone with Avenza for Field Locations

  - Backup Supplies

    - Toolbox with basic tools

    - Bucket

    - Acid washed, HDPE 250 mL bottles as backup for sample collection

  - Post-Field Supplies

    - Vacuum filter apparatus

    - 0.2 um PES filters

    - 0.45 um PES filters

    - Forceps

    - Acid washed, HDPE 30mL bottles for Aqualog Samples

    - Printer labels

    - Field notes

    - Large Beaker

- Pre-Field Preparation

  - Ensure all HDPE bottles are acid washed:

    - Make sure all bottles are free from any and all debris, especially around the threads and cap area. Cleaning the outside of bottles will also help to keep the acid bath at full strength for a longer period of time.

    - Once free from debris, rinse 3x with DI water & let dry.

    - Whatever storage box they’re transported in, make sure it’s also free of debris.

    - If acid bath is prepared, check the pH to ensure it’s below 2. Otherwise refer to the acid washing SOP to create a new bath (T:\Groups\Cellar\Bladon_FEWSLab\Equipment & Forms\SOPs or Methodology\Acid_Washing_Bottles\Acid Bath SOP.doc).

    - Put on PPE (lab coat, long pants, closed toed shoes, latex gloves, face shield, acid gloves).

    - Place the bin of bottles on the cart labeled Richardson 269, remove the top of the field bins and acid bath.

    - Grab one to two 1000 mL beakers (must be glass) and two large watch glasses. Place the beakers in the acid bath, filling up with acid.

    - Slowly add the bottle caps into the beakers, you may need to push them down a little. Once reasonably full, use the 250 mL beaker in the fume hood to ensure the 1000 mL beakers are completely full of acid, and place the watch glass over top, ensure there are no air bubbles around the caps, they should be fully submerged.

    - Next remove one bottle at a time from the field bin, and in one hand, with acid gloves hold the wide part of the bottle with the opening horizontally and gently lower into acid bath filling bottle, do not submerge until almost full to avoid bubbles and splatter. Submerge bottle to completely fill once minimal airspace remains.

    - Repeat for all the bottles.

    - Place the lid on the acid bath make sure not to snap it tight.

    - Let the bottles soak for at least 24 hours.

    - To rinse the bottles, put on all PPE and place medium white bin on the Richardson 269 cart to place the emptied bottles into.

    - Grab the bottles one by one and pour the acid solution smoothly back into the bath. Once empty place in the white plastic bin for rinsing. Repeat for all bottles. Get a separate small bin for the caps and beakers, there should be extra bins in Richardson 273. Remove the watch glasses and place in the bin for rinsing. Then pull the beakers out of the acid, using your acid gloved hand to keep the caps in while draining as much acid as you can from the beaker. Place the entire beaker and caps in the smaller plastic bin.

    - Once all the bottles have been removed from the bath, roll it over to Richardson 273.

    - Carefully add DI water to the beaker with the caps, fill and dump 5 times with DI water, then drain as much water as you can and place the caps in the clean field bin.

      - There should be slightly smaller acid resistant gloves below the sink in Richardson 273 that you can use for rinsing.

    - Remove bottles one at a time from the plastic bin and rinse 3 times each with DI water.

    - As you finish washing a bottle, put on a washed lid on, do not screw on all the way.

    - Then place bottles in a clean field bin to dry, once all the bottles and caps are added, place the lid on top to minimize dust, but leave an opening to allow air to escape. Dump any acid/water from the plastic bin down the sink with water running.

    - Rinse the bucket and gloves and replace in Richardson 269.

    - Clean the face shield with 70% alcohol, properly stow PPE, dispose of latex gloves, and wash hands.

    - Once bottles are dry, place in one of the clear bins with lids to be ready for the field.

  - Update site groups and data sheets as needed

    - If you know there’s sites that are inaccessible, try to reorganize groups a little so we can get to more of the accessible sites.

    - When you move sites around, you’ll need to update the datasheets, site information, and ideally the site groups on the Avenza layers.

      - To make new Avenza layers, go here: T:\Bladon_FEWSLab\10_Katie\1_Research\4_McKenzie_SSN\GIS and load the SSN Sites_2022_12_07.kmz into google earth pro, separate into a folder for each group, download, then upload to tablets.

  - Print off datasheets, SOP, and info on rite in the rain paper via the printer at Oak Creek.

    - See T:\Bladon_FEWSLab\10_Katie\1_Research\4_McKenzie_SSN\Site Groups\Data Sheets  for example data sheets.

    - See T:\Bladon_FEWSLab\10_Katie\1_Research\4_McKenzie_SSN\Site Groups\Guides for example site info sheets.

    - SOP and important info are here: T:\Groups\Cellar\Bladon_FEWSLab\10_Katie\1_Research\4_McKenzie_SSN\Site Groups\SSN Info Summer Samp\Basic Guide Info.doc

    - Print off two datasheets for each group so they have an extra.

  - Print off labels

    - Before sampling, print off enough half-filled labels so you have one per site. Use the Avery non-permanent labels and the Avery software. Labels should have the form MCSN____-YYYYMMDD-_____ with blanks left for the site number and time of sampling.

  - Make sample kits

    - To prep sample kits, place a printed label on a 125mL bottle.

    - Place the bottle, one 0.2um filter, one syringe, and a pair of gloves in a clean quart sized bag. Seal.

    - Make enough kits so you have enough for each site, plus 1-2 extra for each group.

  - Prep extra backup equipment

    - Ensure each group has the following “backup” equipment

      - 250 mL bottles (ideally 1 per site, but at least enough for half the sites)

      - 1 quart bag of extra 0.2 um syringe filters

      - 1 quart bag of 0.45 um syringe filters

      - 1 quart bag of gloves (ideally one with size M and one with size L)

      - 2-4 extra syringes

      - Bag with pencils, sharpies, labeling tape, and zip ties.

      - 1 flat head screwdriver

      - 1 small allen wrench that fits the extendable sampler

  - Just before heading out into the field, place ice packs from the freezer in Oak Creek into coolers.

- Field Procedure

  - To pack the truck, ensure the container with sample bottles or sampling kits, sharpie, labeling tape, and clipboard are in the front of the truck. Pack gloves, syringes, and filters in a backpack to take to the sites. If the bottles aren’t labeled, between sites the person not driving should label the bottles with the sample tape and label the site number, day, (MC#-YYMMDD-) and label as DOC or EEM

  - Locate the field site using the datapoints in Avenza. Before leaving the truck, put on hard hat/helmet and high vis vest (if parking is near the road).

  - On the dataset note the date, weather conditions, and names of researchers. For each site include the time and any observations of the site (flow conditions, landscape conditions, equipment issues, etc.). Also record the bottle number used for DOC and EEM’s.

  - To collect the water sample, ensure the sampling bottle is snuggly attached to the extendable sampler and the sampling head is fully tightened. Find an area of representative flow (no pools), fill the sampling bottle and dump three times to rinse before grabbing a sample.

  - Put on gloves before working with sample bottles or the filter.

  - Rinse the syringe three times with the stream water.

  - Then fill syringe full, screw on the syringe filter. Push 5-10 mL through the filter and discard.

  - Ready the sample bottles by pushing 5-10 mL of the filtered water into the bottle. Cap, shake, and then dump. Repeat three times for each bottle.

  - Fill both 30 mL bottles with filtered water, leave a bit of space at the top. For the DOC samples they should be pretty close to the top Cap and finish the sample label with the time of sampling (MC#-YYMMDD-HHMM). Place in the cooler.

    - NOTE: Label the time in PST which is an hour earlier during daylights savings time (March to November)

  - Make sure you’ve taken pictures of the site, you’ve got both samples, and all the trash from the filtering.

  - Repeat for all sample sites.

- Post-Field Procedure

  - Once back from the field, all the samples should be placed in the fridge in Richardson 278 in a clear bin on the Bladon-FEWS-PNNL-DOM shelf. Analysis of the samples with the Aqualog should be performed within 2 weeks, but ideally the day or two following collection.

  - Ensure all equipment is clean, dry, and placed in its storage location.

  - Take photos of the final datasheets and any other field notes and upload to T:\Groups\Cellar\Bladon_FEWSLab\Research\PNNL_DOM\Field Notes\YYYY_MM_DD\

  - Upload any photos taken to T:\Groups\Cellar\Bladon_FEWSLab\Research\PNNL_DOM\Photos\YYYY_MM_DD\

  - Ensure the tablet is plugged in, truck keys are in Oak Creek, and the doors are locked.

  - The next day prep DOC samples need to be sent to the PNNL.

    - Email Sammi Grieger to let her know samples are being sent and double check she has time to analyze them (samantha.grieger@pnnl.gov).

    - Find the small cooler in the Oak Creek lab, place 3 medium ice packs in the cooler.

    - Ensure all the caps are tightly on the bottles, and pack into the cooler.

    - Place packing materials in any open space so nothing is moving around.

    - Place the lid on the cooler, and tape around four times to secure.

    - Take to the OSU printing and mailing center (4700 SW Research Way, Corvallis, OR 97333). Use the second driveway.

    - Ask to overnight the samples (timing is more important than cost) using the PNNL index (NL419A) to Sammi Grieger.

      - Sammi Grieger, MCRL-219, Pacific Northwest National Laboratory, 1529 West Sequim Bay Road, Sequim, WA 98382.

      - Phone number is 360-582-2555

- Consumables

  - Filters: Whatman Puradisc 25 mm PES Syringe Filters, 0.2 µm, 50 Pack, 6780-2502

  - Syringes: Fisherbrand 60 mL sterile syringes

    - https://www.fishersci.com/shop/products/sterile-syringes-single-use-12/14955461

  - Labels

    - EPA:

    - OSU:

  - Vacuum Filters

    - 0.2um:

    - 0.45um:

- References

  - O’Donnell, J. A. 2017. Standard Operating Procedure: Chemical Characterization of Surface Water, Version 1.0. Stream and Lake Communities and Ecosystems Monitoring Protocols, Arctic Inventory and Monitoring Network. National Park Service, Fairbanks, Alaska. Available online at https://irma.nps.gov/DataStore/DownloadFile/605931.

  - Motter, K. and Jones, C. 2010. Standard Operating Procedure for the Analysis of Dissolved and Total Organic Carbon CCAL 20A.3. Cooperative Chemical Analytical Laboratory, College of Forestry, Oregon State University, Corvallis, Oregon. Available online at https://ccal.oregonstate.edu/sites/ccal/files/sop/CCAL%20OC%20SOP%202015.pdf.

- Document Revision History

  - Original Document: June 2022

  - Revised 2022-08-01:

    - Included shipping directions for DOC samples and updated equipment.

  - Revised 2024-02-09:

    - Updated the post-collection directions to include more info.