# Storm chasing field manual

samples retrieval and checkups

Contents

Materials checklist	2

Instruments	2

General	3

Procedure	4

Before leaving	4

In the field	4

ISCO	4

Sondes	10

Pressure transducer	11

Grab sample	11

# Materials checklist during checkups for the storm campaign

### Supplies

| Sampling: - dirty or not acid washed ISCO bottles and bottle lids - acid washed ISCO bottles and bottle lids - 8 1L acid washed bottles - Extended sampler - Coolers with ice packs - Lables for grab sample bottle ISCO: - ISCO - ISCO battery - ISCO tubing - ISCO battery leash - Battery plastic box - ISCO tubing (internal and external tubes) - Replacement deep cycle batteries - Hose clamps - Flat head screwdriver or socket driver for hose clamps - Long tape measure - Zip ties - Diameter tape measure - Voltmeter - 1000 ml graduated plastic cylinder - ISCO calibration bottle - Soap Pressure transducer: - Laptop - Cable for readout and launching of level logger (when bottles are retrieved) - Cable for readout and launching of baro logger (when bottles are retrieved) Sondes: - Sonde handheld - Sonde field cable - D batteries (x4 for each sonde + extras) | Rainfall gauge - Coin cell battery - Small screwdriver for battery replacement - Cable for readout (when bottled are retrieved) Miscellaneous: - Keys - umbrella - Nitrile gloves - Waders and/or boots-rubber boots - Knife - Duct tape - Tablet with georeferenced maps - Paper maps - Permits, permissions, contact info - Colorful ribbon for marking - Field notebooks - Field forms - Extra rain on paper - Washi labeling tape - Sharpies and pencils - Tool box - Pulaski, shovel, saw (for clearing debris off road. Required during fire season). - Water pump backpack (summer field work) - 5 gallon jug filled with potable water (summer field work) Safety: - Safety glasses - Work gloves - Spot unit - Ear protection - Hard hat - First aid kit |
| --- | --- |

### General

| Tools: - Crimpers - cable cutters (small) - Bolt cutters (big cable cutter) - Plastic mallet (plastic hammer) - Sawzall (reciprocation Saw ) - Socket wrench (find large drive wrench) - Hose clamp socket - Epoxy gun Hardware: - 1/4" coated cable - 3/16” coated cable - Small and large cable u-bolts - 3/16” ferrules - Hose clamps - Carabiners (1x sonde) - Platers - Socket for u bolt |  |
| --- | --- |

# Procedure

### Before leaving

Two or three days before leaving:

- Reserve the trucks at motorpool needed for sampling.

### If needed, upload maps in Avanza in all the tablets.

- Check the weather for windy conditions and fire danger (NOAA is a good source for wind and snow levels)

- Print field forms on write-in-rain paper (T:\Groups\Cellar\Bladon_FEWSLab\Research\Wildfire_Water_Security\02_Nodes\01_Empirical\05_Projects\storm chasing\03_Forms\Field_forms.doc).

- Double-check that you have everything on the material checklist above and don’t forget the lock keys!

Check out text: Send a group text in TEAMS to the let group know your general plan and your estimated time of return

## In the field

## Good practice: say out loud all the steps you are taking in the field.

- Take a general look around the site – are there any new failures on the slopes or banks? Are there any new trees or rootwads that have fallen over?

- Fill out the general information on the “field_form_stormchasing.pdf” (file location: “T:\Research\Wildfire_Water_Security\02_Nodes\01_Empirical\05_Projects\storm chasing\Lane1\forms”)

  - Record the site name (as shown on the map)

  - Record the date in the yyyy-mm-dd format

  - Record the time as 00:00 format, in Pacific Standard Time

  - Record the field crew members’ (OBS = observer) initials

  - Record the weather (WX) – Is it raining? Is it clear and sunny? Is it windy?

### Grab sample

      - Label the sample: XXXXXXX.

      - Check the time, if there is time between the sonde sampling, collect the grab sample, if not wait for the sonde to finish the sampling.

      - Record in the field notes the name of the sample, the bottle number and the day and hour you collected the sample.

      - Rinse the bottle of the extendo sampler 3 times with the creek water

      - Rinse the 1L sample bottle 3 times with the creek water from the extendo sampler

      - Fill the 1L sample bottle with creek water leaving no headspace.

      - Place the bottle in the cooler.

### ISCO

- Check the ISCO screen and record in the field notes the information displayed.

- ISCO intake line submerged? Check to see if the intake line is submerged or possibly buried by sediment. Take notes.

- Record any problem encountered, is the tube still attached? Did the samples overflow? Did the program crash?

  - If the ISCO displays “Sample missed” it is likely because either the intake line is not submerged (see step 17) or the tubing has come off at the ISCO.

  - If the ISCO displays “Pump Jam failure” you’ll need to replace the ISCO with the spare as we are not able to fix this issue in the field.

- Record the time in your field notes.

- Before opening the ISCO, make sure you have enough time before the next sampling, if not, wait for the ISCO to sample.

- Wear nitrile gloves and open the ISCO.

- If raining, use the umbrella to cover the bottles, no rain should enter the bottles.

- Empty the water that is possibly present in the plastic container of the battery and on the ISCO itself.

- Place the lid on the bottles with samples and replace them with the dirty ones, make sure you take out the lids.

- Make sure the ISCO program is still on and record the hour and date of the next sampling. Take a picture of the screen and read out loud what it is written.

- Record the number of bottles and the bottle numbers in the field form and place the bottles in the cooler.

- Battery case: carefully empty any water present in the case.

- Battery voltage: Open up the ISCO battery housing. Use the voltmeter to check the voltage of the ISCO battery and record. We will consider change out the battery if the voltage starts to read ~12.15 volts.

### Sondes

- Before retrieving the sonde from the water, make sure you have enough time from the next sampling.

- Partially download the data in the sonde with the sonde handheld.

- Check the battery level and record it.

- Record the next sampling time.

- Check if the sonde is launched.

- M

- ake sure the red light on the sonde is pulsing, that means the sonde is launched and ready to work!

- Slide the sonde down the pipe so that it reaches the bottom and is submerged.

  - Note: You may have to push the cable a bit, shimmy the sonde, or pull it back out and retry with more momentum if it gets stuck.

- Use the long-shackled padlock or bike lock to lock up the end of the tube for theft prevention.

### Back to the lab:

- Back to the lab, place the bottles in the fridge room in Richardson.

- The next day or within 48 hours filter the samples as described in the lab sop.

### Grab sample

      - Rinse the bottle of the extendo sampler 3 times with the creek water

      - Rinse the 1L sample bottle 3 times with the creek water from the extendo sampler

      - Fill the 1L sample bottle with creek water leaving no headspace.