# FEWS Acid Washing SOP

FEWS Laboratory, PI: Kevin Bladon

Ryan Cole, Kate McCredie, Emily Nussdorfer

Introduction:
Acid washing bottles is a necessary step to maintain high confidence in the accuracy of water collection sample. However, acids are corrosive substances which cause destruction of living tissue by chemical action at the contact site causing burns. The fumes of the acid bath may also cause destruction of the tissue of the mucous membranes and upper respiratory track. For these reasons, it’s important to always wear full PPE at all times when handling or using the acid bath. Always add acid to water ("do as you otta, add acid to wata”).

Table of Contents (clickable on word)

PI Approval and Safety Contacts	3

Safety Contacts	3

Personal Safety	4

Chemical/Hazardous Substances	4

RAMP – Risk Assessment Process	4

Personal Protective Equipment (PPE)	5

For acid washing	5

For preparing or neutralizing acid bath:	5

Safety controls	5

Engineering Controls- fume hood	5

Administrative Controls	5

Emergencies 	6

First Aid Procedures	6

Chemical Spill	6

Other Emergencies	7

Equipment and Supplies	8

Chemicals	8

Equipment	8

For acid washing	8

For making the acid bath	8

Special Storage & Handling Requirements	9

Storage:	9

Transporting:	9

Protocol Procedure	10

Housekeeping	10

QA/QC	10

Cleaning of Bottles/Glassware	10

Pre-wash rinsing	10

Washing	10

Post-wash rinsing and drying	11

Preparing Acid Bath	11

Neutralization and cleanup of the Acid Bath	12

Appendix A - References	13

Appendix B – Acid Bath Log Sheets	13

# PI Approval and Safety Contacts

| Lab Name: | FEWS Laboratory |
| --- | --- |
| Principal Investigator: | Kevin Bladon |
| Date SOP was approved: | 2024-11-04 |
| Name of employee responsible for training others on this SOP: | Ryan Cole, Kate McCredie, Emily Nussdorfer |
| Location(s) covered by this SOP: | Richardson 269, 271, 273 |

Principal Investigator SOP Approval

By signing and dating here the designee certifies that the Standard Operating Procedure (SOP) for Insert SOP Name is accurate and effectively provides standard operating procedures for laboratory personnel.

___________________________________________________________________________________

Signature				Printed Name/Title					Date

## Safety Contacts

Ryan Cole – 281-798-6528, ryan.cole@oregonstate.edu

Kate McCredie – 971-998-4058, kate.mccredie@oregonstate.edu

Environmental Health and Safety - ehs@oregonstate.edu

Hazardous Waste Pickup - https://ehs.oregonstate.edu/waste

Public Safety Dispatch – 541-737-3010

In case of Emergency call 911

# Personal Safety

## Chemical/Hazardous Substances

| Chemical Name | CAS # | Hazards | Permissible Exposure Limit (PEL) |
| --- | --- | --- | --- |
| Hydrochloric acid | 7647-01-0 | Corrosive to respiratory system, skin, eyes, and lungs. | 5 ppm (OSHA) |

## RAMP – Risk Assessment Process

| Recognize the Hazard | Assess Risk (based off severity and likelihood) | Mitigate Hazards | Emergency Procedures |
| --- | --- | --- | --- |
| Corrosive chemical burn or eye damage from hydrochloric acid spill | Medium (likelihood is low with PPE, severity is medium) | Use dilute hydrochloric acid. Use of acid-resistant gloves, face shield, goggles, and FRCP lab coat. In person training of process, | If Hydrochloric acid gets on skin or eyes flush eyes or skin for 15 min at a safety shower or eyewash. See a physician. Alert supervisor to fill out paperwork. Supervisor to alert EHS. |
| Inhalation of hydrochloric acid vapors | Low (likelihood is low, consequence is medium) | Use dilute hydrochloric acid, perform all work in fume hood raised to 18 in. Allow acid washed bottles to fume off by pouring DI water over and leaving in the fume hood for 10+ mins. | Remove from area into fresh air. Seek medical attention. Notify supervisor immediately. |

## Personal Protective Equipment (PPE)

### For acid washing

- Before coming to the lab: Wear closed-toed shoes and long pants.

- Fire/Chem Protection (FRCP) Lab Coat – the Blue one

- Disposable nitrile gloves (under acid resistant gloves)

- Acid resistant turquoise nitrile gloves (Lime green Solvex 37-175 – Type A JKLOPT)

- Safety goggles

- Face shield

- Optional: ABEK Respirator

### For preparing or neutralizing acid bath:

- Everything listed above

- Chemical/Splash apron

- A co-worker to assist

# Safety controls

## Engineering Controls- fume hood

NOTE: Fume hood should be raised to 18 inches and ventilation lever at back of hood should be set to downflow for best ventilation. DO NOT acid wash using the side sliders to open the fume hood.

## Administrative Controls

- In person training on acid bath procedures – Contact Ryan Cole or Kate McCredie to set up training. Be sure to sign off on your training.

- DO NOT work alone when preparing or neutralizing an acid bath.

- Raise any safety concerns you may have with project leads and/or Environmental Health & Safety (ehs@oregonstate.edu)

# Emergencies

## First Aid Procedures

If an accident happens, supervisors are required to file an online OSU HR Advocate Public Incident Reporting Form within 24 hours of the incident via the Insurance and Risk Management Website. Additional forms may be required based on care needed.

If inhaled

Remove from area into fresh air. Seek medical attention. Notify supervisor immediately.

In case of skin contact

Remove all contaminated clothing and rinse the affected area with running water from eyewash for 15 minutes or use the emergency shower for at least 15 minutes. Seek medical attention. Notify supervisor immediately.

Location of nearest Safety Shower: Outside of lab in hallway between RH269 and RH271

In case of eye contact

Immediately rinse the eyeball and inner surface of eyelid (by forcibly holding the eye open) with copious amounts of water from the emergency eyewash station for at least 15 minutes. Seek medical attention. Notify supervisor immediately.

Location of nearest eyewash(s): Co-located with sink in RH269

Co-located with sinks in RH271

If ingested

After swallowing: drink water (two glasses at most), avoid vomiting (risk of

perforation). Call a physician immediately. Do not attempt to neutralize.

## Chemical Spill

OSU Chemical Spill Safety Instruction: http://ehs.oregonstate.edu/sites/ehs.oregonstate.edu/files/pdf/si/spill_response-chemicals_si.019.pdf

Location of Spill Kit: On counter opposite of fume hood with acid baths.

General Guidelines

For spills less than 1 gallon in size, low hazard chemicals:

If the spill is from a small, low hazard chemical, the lab may clean-up the spill themselves if trained and they feel comfortable to do so. Follow all chemical spill response guidelines here: http://ehs.oregonstate.edu/sites/ehs.oregonstate.edu/files/pdf/si/spill_response-chemicals_si.019.pdf and always report all spills to EH&S via SciShield ObserveNow.

For spills greater than 1 gallon in size, high hazard chemicals:

If a chemical spill is greater than 1 gallon in volume or is a particularly hazardous material (strong acid or base, carcinogen, highly reactive chemical, etc.), call Public Safety (541-737-7000), and tell them to contact the on-call EH&S personnel to respond to the spill.

Follow all chemical spill response guidelines here: http://ehs.oregonstate.edu/sites/ehs.oregonstate.edu/files/pdf/si/spill_response-chemicals_si.019.pdf

## Other Emergencies

Medical Emergency Dial 911

Life Threatening Emergency, After Hours, Weekends and Holidays – Dial 911 (This will connect you to Good Samaritan Hospital Corvallis where they will be able to treat the victim).

Fire – If trained and comfortable to do so, attempt to extinguish the fire with the appropriate extinguisher using the PASS method. Call 911. If unable to extinguish not comfortable, pull the fire alarm, alert others, evacuate and call 911.

# Equipment and Supplies

## Chemicals

  - Sodium Bicarbonate (large bucket should be located to the right of the fume hood)

  - 1 bottle of the glass 2.5L concentrated (37 % weight/weight) HCl bottles from the OSU Chem Store per 20 L bin – ONLY WHEN MAKING NEW ACID BATH

  - 10 % v/v Hydrochloric acid for cleaning – diluted from concentrated 37 % weight/weight in large tub.

## Equipment

### For acid washing

  - PPE

  - pH Strips (0-4 pH ranges)

  - Acid Bath Logbook

  - DI Rinse catch basin

  - Transport container for catch basin

  - DI Rinse 1 liter bottle or wash bottle

  - Sodium bicarbonate: for neutralizing small spills

  - Large beaker and small beaker (for pouring acid into bottles or holding glass stoppers)

  - Watch glass (for placing over large beaker to keep caps from floating)

### For making the acid bath

  - PPE

  - Stirring Mechanism (rod used to collect stir bars, see RH 273 in drawer across from fume hood)

  - Beaker to measure acid volume

  - Sterilite tubs with lids

  - pH strips and long tweezers

# Special Storage & Handling Requirements

## Storage:

- Store 10% v/v hydrochloric acid baths in fume hood with lid tightly closed and hood door closed when not in use.

- Store concentrated 37% w/w hydrochloric acid in “Acid Storage” cabinet underneath the fume hood in Richardson 269.

## Transporting:

DO NOT transport acid baths! Leave them in the fume hood.

Treat any glassware or bottles removed from the acid bath as hazardous until 3x rinses are completed. Transport them in a secondary container on a cart to RH273 for the final rinses.

Transport all concentrated 37% w/w hydrochloric acid in a secondary bottle container.

# Protocol Procedure

## Housekeeping

- For any small acid spills (<50mL): (wearing PPE) neutralize first by sprinkling baking soda over until it stops reacting. Use DI water and paper towels to clean up the rest. Discard in the trash.

## QA/QC

- Once a week, test the pH with the testing strips to make sure it remains at ~1pH. Replace acid bath if it reaches >2 pH.

    - Can dip pH strip into acid bath using a gloved hand while wearing PPE. Only dip one end of strip. Rinse strip with DI and dispose in trash can.

- Make sure the outside of bottles are also cleaned of debris, otherwise an Electrical Conductivity meter will need to be used to make sure the acid bath remains at full strength. An EC meter is also needed if you want data to pass EPA-level QA/QC standards.

## Cleaning of Bottles/Glassware

Note: see appendix for detailed information on this process and strategies.

### Pre-wash rinsing

- Don disposable nitrile gloves.

- Prior to rinsing: remove sharpie (not the bottle number unless otherwise indicated), tape, and any debris from the items. If there are any metal components or foam in the lids remove those. Only glass and certain plastics can be placed in the acid bath.

- Remove dirt and debris from the items. Clear bottle threads and caps of residue. Use a bottle brush or scrub brush as needed. The bath will need to be replaced more frequently if detritus is introduced to the bath.

- Rinse each item 3x with DI water inside and outside. Shake out excess water before placing in a “rinsed for acid washing” bin. Use a cart for transporting.

### Washing

- Don all PPE required.

- Lift the fume hood vertically, keeping the horizontally sliding panels closed. Adjust the height to a comfortable working height, ensuring it is below the maximum height of 18 inches.

- To avoid acid dripping onto the floors and the outside of the fume hood place your “to be acid washed” items inside the fume hood before starting.

- Slowly submerge the items and avoid splashing. Avoid bubbles to a reasonable degree. Make sure to fully coat the item in the acid. If it gains a little air post submersion, that is fine since it is still in an acidic environment. When done close the lid on the acid bath.

- Close the fume hood. With an erasable marker write the time/date and your contact. The items can be taken out twelve hours after submersion.

- Rinse any other items that have touched acid with DI water. Clean any surfaces that have minor spills on them. Rinse the acid-resistant gloves with DI water and pat dry before putting away.

### Post-wash rinsing and drying

- Don all PPE required.

- Place rinse container (clear labeled bin) in the fume hood to hold the empty acid washed bottles in. There should be a small and larger bin in 269.

- Slowly empty the bottles to avoid splashing. Place in rinse bin.

- Pour DI water (brown 1L labeled container) over the items in the rinse bin.

- Let the bin and items sit in the closed fume hood for at least ten minutes.

- Re-don any PPE. Remove the rinse bin and place it in a labeled transport bin on a cart. Avoid splashing any of the acidified rinse water while transporting.

- Continue to treat all the items and the rinse bin as hazardous up until everything is finished being rinsed.

- Rinse each item 3x with DI water inside and outside. Shake out excess water.

- Place items on an acid washing only drying area. Write the date and your contact on the whiteboard next to the sink.

- When dry, cap bottles and store in a labeled container.

## Preparing Acid Bath

- Find a buddy! Working with large volumes of acid is safer in pairs.

- Put on all PPE – including splash apron

- Clean and rinse with DI an acid-resistant container (e.g. Sterilite) to be used for the acid bath.

- Find a suitable location (fume hood) the water bath can remain for long periods of time because once it’s filled up, it should not be moved.

- Find out how much volume of water can be held when the container is ½ filled up. Make a mark on the container for ½ way.

  - That volume will be your guide for the 10% volume/volume calculation.

  - Using a cleaned and rinsed hydrogen peroxide container (4 liter) can be a useful way to obtain a more accurate measure of water being added to your acid bath.

  - If your tub can hold 20 liters at the ½ way mark, you will add in 18 liters of DI and 2 liters of concentrated HCl to get the 10% v/v HCl solution.

- Once doing the calculation for your water:HCl solution, mark again on the tub where approximately the level of water is needed (example – mark at 18 liters for a 20 liter acid bath).

- Add the necessary amount of water.

## Neutralization and cleanup of the Acid Bath

- Wear all PPE

- Neutralize the acid bath by adding one cup of sodium bicarbonate one cup at a time until bubbling completely stops. Be sure to break up baking soda chunks before adding to the acid to prevent acid splashes.  For a 20 liter 10% v/v HCl acid bath it will take approximately 1.3 L of sodium bicarbonate.

- Stir with the stirring rod to ensure that the sodium bicarbonate does not settle at the bottom of the tub.

- Check pH with the pH strips to reach 7. Continue adding sodium bicarbonate until pH 7 is reached (usually the lack of bubbling is a good sign you’re getting close). Make sure you log the pH of the dumped waste in the acid bath logbook!

- Now that the water has been neutralized, all you have is slightly salty water that is safe to dump down the drain.

- Do not pick up the full tub of neutralized water – the weight of the water will likely cause a failure of the tub causing a large and messy spill. Instead, use a smaller tub and a bottle to help transfer water from the large tub to the sink.

- Pour the neutralized water down the sink with the tap water running to dilute the chlorine further.

- Continue bailing water from the main acid bath until it’s safe to transport the larger acid bath container to the sink itself.

- Using DI water, rinse off all the walls of the acid bath. Ensure no debris are left in the corners & no cracks or points of weakness are visible on the acid bath. If they are found, replace immediately.

- You’re now ready to refill the acid bath.

# Appendix A - References

Emily’s Acid Washing Strategy guide™ Acid washing strategy guide draft.docx

(It’s still messy and in progress)

https://ehs.oregonstate.edu/sites/ehs.oregonstate.edu/files/pdf/si/acid_bath_safety_si.pdf (OSU EHS for acid bath)

https://www.fishersci.ca/shop/msdsproxy%3FproductName%3DA144500%26productDe (SDS)

https://www.columbuschemical.com/MSDS/SDS/Hydrochloric%20Acid%2010%25%20(v-v)%208780.pdf (SDS closer to the percent of HCL)

https://www.osha.gov/chemicaldata/620 (PEL)

# Appendix B – Acid Bath Log Sheets

Acid Bath TESTING Log -- Richardson Hall 269 Fume Hood

| Date | Contact Name | Bath number | Activity? (Making acid bath, testing pH, other) | pH Strip Reading | EC meter Reading |
| --- | --- | --- | --- | --- | --- |

Acid Bath CLEANING Log -- Richardson Hall 269 Fume Hood

| Date | Contact Name | Bath number | Approximate volume Sodium Bicarbonate | Final pH Reading | EC meter Reading |
| --- | --- | --- | --- | --- | --- |

FEWS Lab Acid Bath Training Confirmation

I have been trained on the safe and proper use of the acid bath, and I am informed of the location and availability of the Acid Bath Standard Operating Procedures and Safety Data sheets:

| Name | Signature | Date |
| --- | --- | --- |