638-94829

Apr. 2021

LabSolutions™ DB/CS

Instruction Manual

System Users Guide [LabSolutions TOC Volume]

Read this manual thoroughly before you use the product.

Keep this manual for future reference.

This page is intentionally left blank.

Introduction Read this Instruction Manual thoroughly before using the product.

Thank you for purchasing this product.

This manual describes the operation and options for this product. Read this manual thoroughly before using the product and operate the product in accordance with the instructions in this manual.

Keep this manual for future reference.

Important

• If the user or usage location changes, ensure that this manual is always kept together with the product.

• If this manual or a product warning label is lost or damaged, immediately contact your Shimadzu representative to request a replacement.

• To ensure safe operation, contact your Shimadzu representative if product installation, adjustment, re-installation (after the product is moved) is required.

Notice

• Information in this manual is subject to change without notice and does not represent a commitment on the part of the vendor.

• Any errors or omissions which may have occurred in this manual despite the utmost care taken in its production will be corrected as soon as possible, although not necessarily immediately after detection.

• All rights are reserved, including those to reproduce this manual or parts thereof in any form without permission in writing from Shimadzu Corporation.

• LabSolutions is a trademark of Shimadzu Corporation.

Windows is a registered trademark of Microsoft Corporation in the United States and/or other countries.

Third-party trademarks and trade names may be used in this publication to refer to either the entities or their products/services, whether or not they are used with trademark symbol "TM" or "®".

© 2021 Shimadzu Corporation. All rights reserved.

Original version is approved in English.

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] i

Introduction

Indications Used in This Manual The following symbols are used in this manual:

Indication Meaning

Emphasizes additional information that is provided to ensure

the proper use of this product.

Indicates the location of related reference information.

Mouse and Window Operations

Description Meaning

Click Means pointing to an object and pressing the left mouse button once.

Right-click Means pointing to an object and pressing the right mouse button once.

Double-click Means pointing to an object and pressing the left mouse button twice.

The names of buttons in a window, and window names, are

Text in square brackets [ ]

Text in quotation marks " "

enclosed in square brackets.

Example: Click [OK].

Entered numerical values, text, and keyboard key names are indicated enclosed in quotation marks.

Example: Press the "N" key while holding down the "Ctrl" key.

ii LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]

Use of Instruction ManualsUse of Instruction Manuals

Shimadzu TOC analyzers include the LabSolutions TOC instrument control software and the LabSolutions Manager data management software as the LabSolutions system. The instruction manuals, including this manual, are organized as follows.

For details on how to operate the TOC analyzer, refer to the instruction manual for your particular model.

Instruction Manual Description

LabSolutions Manager DB Installation &

Maintenance Guide

or

LabSolutions Manager CS Installation & Maintenance Guide

LabSolutions DB/CS Installation & Maintenance Guide [LabSolutions TOC Volume]

LabSolutions Manager DB System Users Guide

or

LabSolutions Manager CS System Users Guide

LabSolutions DB/CS System Users Guide [LabSolutions TOC Volume] (This manual)

This manual explains software installation, troubleshooting, and environment settings for the LabSolutions system.

This manual explains LabSolutions TOC model-specific installation, troubleshooting, and environment settings.

This manual explains how to use the LabSolutions system management function.

This manual explains how to use LabSolutions TOC model-specific management functions.

For details on each LabSolutions command and execution method, refer to the provided Help file.

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] iii

Introduction

Warranty Shimadzu provides the following warranty for this product.

1. Period:

Please contact your Shimadzu representative for information about the period of this warranty.

2. Description:

If a product/part failure occurs for reasons attributable to Shimadzu during the warranty period, Shimadzu will repair or replace the product/part free of charge. However, in the case of products which are usually available on the market only for a short time, such as personal computers and their peripherals/parts, Shimadzu may not be able to provide identical replacement products.

3. Limitation of Liability:

(1) In no event will Shimadzu be liable for any lost revenue, profit or data, or for special, indirect, consequential, incidental or punitive damages, however caused regardless of the theory of liability, arising out of or related to the use of or inability to use the product, even if Shimadzu has been advised of the possibility of such damage.

(2) In no event will Shimadzu's liability to you, whether in contract, tort (including negligence), or otherwise, exceed the amount you paid for the product.

4. Exceptions:

Failures caused by the following are excluded from the warranty, even if they occur during the warranty period.

(1) Improper product handling

(2) Repairs or modifications performed by parties other than Shimadzu or Shimadzu designated companies

(3) Product use in combination with hardware or software other than that designated by Shimadzu

(4) Computer viruses leading to device failures and damage to data and software, including the product's basic software

(5) Power failures, including power outages and sudden voltage drops, leading to device failures and damage to data and software, including the product's basic software

(6) Turning OFF the product without following the proper shutdown procedure leading to device failures and damage to data and software, including the product's basic software

(7) Reasons unrelated to the product itself

(8) Product use in harsh environments, such as those subject to high temperatures or humidity levels, corrosive gases, or strong vibrations

(9) Fires, earthquakes, or any other act of nature, contamination by radioactive or hazardous substances, or any other force majeure event, including wars, riots, and crimes

(10) Product movement or transportation after installation

(11) Consumables and equivalent items

Recording media such as CD-ROMs are considered consumable items.

* If there is a document such as a warranty provided with the product, or there is a separate contract agreed upon that includes warranty conditions, the provisions of those documents shall apply.

iv LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]

Software License AgreementSoftware License Agreement

PLEASE READ THIS AGREEMENT CAREFULLY BEFORE USING THE SOFTWARE. SHIMADZU Corporation ("SMZ") is willing to license the SMZ software provided herein, together with accompanying documentation (collectively "SOFTWARE") to you only upon the condition that you accept all of the terms and condition contained in this License Agreement. By using the SOFTWARE, you agree to be bound by the terms of this Agreement. If you do not agree to all these terms of this Agreement, promptly return the unused SOFTWARE to the party (either SMZ or its reseller) from whom you acquired it to receive a refund of the amount you paid.

1. License

SMZ grants you a non-exclusive and nontransferable license to use the SOFTWARE subject to the following terms and conditions.

2. Limitation of Use

Except as specifically authorized by SMZ, you may NOT:

(1) Use the SOFTWARE, or permit the SOFTWARE to be used, on more than one computer at any one time.

(2) Copy the SOFTWARE except one (1) archival copy of the SOFTWARE.

(3) Modify, reverse engineer, decompile, disassemble, or create derivative works based upon the SOFTWARE.

(4) Transfer, rent, lease or grant any rights in the SOFTWARE in any form to anyone else.

3. Title and Ownership

This license is not for sale and it may not be assigned or sublicensed to anyone else. Title and all associated intellectual property rights to the SOFTWARE shall remain in SMZ and/or its licensor.

4. Upgrades

You are entitled to receive all official software upgrades for the SOFTWARE that SMZ will release as deemed necessary by SMZ. An upgrade means certain supplemental program modules and/or information for bug fixing and/or updates to the defects and/or failures of the SOFTWARE that are acknowledged or confirmed by SMZ. An upgrade excludes hardware, network, consulting services, third party products, operation and general computer system maintenance. All supplemental program module for upgrades and enhancements furnished to you shall be deemed to be part of the SOFTWARE and subject to the terms and conditions set forth in this Agreement.

5. Limited Warranty

SMZ warrants that for a period of one (1) year from the date of purchase, as evidenced by a copy of the receipt, the media on which SOFTWARE is furnished will be free of defects in materials and workmanship under normal use.

Except for the foregoing, SOFTWARE is provided AS IS. Your exclusive remedy and the entire liability of SMZ and its suppliers under this limited warranty will be, at SMZ's option, repair, replacement, or refund of the Software if reported (or, upon request, returned) to the party supplying the SOFTWARE to you. In no event does SMZ warrant that the Software is error free or that you will be able to operate the SOFTWARE without problems or interruptions.

EXCEPT AS SPECIFIED IN THIS WARRANTY, ALL EXPRESS OR IMPLIED CONDITIONS, REPRESENTATIONS, AND WARRANTIES INCLUDING, WITHOUT LIMITATION, ANY IMPLIED WARRANTY OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE, NONINFRINGEMENT, ARISING FROM A

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] v

Introduction

COURSE OF DEALING, USAGE, OR TRADE PRACTICE, ARE HEREBY EXCLUDED TO THE EXTENT ALLOWED BY APPLICABLE LAW.

6. Limitation of Liability

IN NO EVENT WILL SMZ BE LIABLE FOR ANY LOST REVENUE, PROFIT OR DATA, OR FOR SPECIAL, INDIRECT, CONSEQUENTIAL, INCIDENTAL OR PUNITIVE DAMAGES, HOWEVER CAUSED REGARDLESS OF THE THEORY OF LIABILITY, ARISING OUT OF OR RELATED TO THE USE OF OR INABILITY TO USE SOFTWARE, EVEN IF SMZ HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGE. IN NO EVENT WILL SMZ'S LIABILITY TO YOU, WHETHER IN CONTRACT, TORT (INCLUDING NEGLIGENCE), OR OTHERWISE, EXCEED THE AMOUNT YOU PAID FOR SOFTWARE.

7. Termination

This License is effective until terminated. You may terminate this License at any time by destroying all copies of SOFTWARE including any documentation. This License will terminate immediately without notice from SMZ if you fail to comply with any provision of this License. Upon termination, you must destroy all copies of SOFTWARE.

8. General

(1) This Agreement is the entire agreement. If any provision of this agreement is held invalid, the remainder of this agreement shall continue in full force and effect.

(2) This Agreement shall be construed and governed in accordance with the laws of Japan, excluding its conflict of law rules.

(3) The exclusive jurisdiction for any disputes arising out of or in connection with this Agreement shall be Kyoto District Court of Japan.

(4) The invalidity or unenforceability of any provision of this Agreement shall not affect the validity or enforceability of any other provision.

After-Sales Service and Availability of Replacement Parts ■ After-Sales Service

If any problem occurs with this product, perform an inspection and take appropriate corrective action as described in the "LabSolutions Manager DB (CS) Installation & Maintenance Guide", "LabSolutions DB/CS Installation & Maintenance Guide (LabSolutions TOC Volume)", and Help file.

If the problem persists, or the symptoms are not covered in the troubleshooting section, contact your Shimadzu representative.

■ Replacement Parts Availability

Replacement parts for this product will be available for a period of seven (7) years after the product is discontinued. Thereafter, such parts may cease to be available. Note, however, that the availability of parts not manufactured by Shimadzu shall be determined by the relevant manufacturers.

vi LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]

Contents

Introduction Indications Used in This Manual ......................... ii Mouse and Window Operations .......................... ii

Use of Instruction Manuals ................................ iii

Warranty .......................................................... iv

Software License Agreement ............................... v

After-Sales Service and Availability of

Replacement Parts ............................................. vi

1System

Administration

1.1 System Administration Policy Settings ................... 1 1.1.1 Instrument Policies .............................................. 1 1.1.2 Audit Trail ....................................................... 3

1.2 Numerical Rounding and Number of Displayed Digits ................................................................. 6

1.3 System Operation ................................................ 8 1.3.1 Screen Lock to Protect System ................................. 8

2 Data Management 2.1 Data Management Using DataManager ............... 10 2.1.1 LabSolutions TOC File Formats ............................... 10

2.2 Other Functions in DataManager ........................ 13

2.2.1 Importing Local Files in the PC to the Database ............ 13

2.2.2 Data Set and Result File Operations ......................... 13

2.2.3 Relating the File Type of Registered Data to Programs ..... 14

3History

Management

3.1 Creating History ................................................ 15

3.2 Checking the Change History of a Sample Table ................................................................ 16

3.3 Checking the Change History of Measurement Parameters ........................................................ 17

3.3.1 Change History of a Measurement Parameter File in the Measurement Application ................................. 17

3.3.2 Change History of a Measurement Parameter File in a Project ......................................................... 19

3.4 Checking the Change History of the System Settings File ...................................................... 21

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] vii

This page is intentionally left blank.

1 System Administration This chapter explains the system administrator functions that are specific to the TOC

1

model.

For details on other functions, refer to the LabSolutions Manager CS or LabSolutions

Manager DB System Users Guide.

1.1 System Administration Policy Settings

For details, refer to "■ Instrument Policies" in "1.2.1 Set System Administration Policy

(Security Policy)" in the LabSolutions Manager CS or LabSolutions Manager DB System Users Guide.

1.1.1 Instrument Policies

1 Double-click the (Security Policy) icon in [Administration] of the [LabSolutions Main] window.

2 Click the [Instrument] tab.

3 Select [TOC] from the [Instrument Type] list.

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] 1

1 System Administration

4 Set each item and click [OK].

| Name | Function |
| --- | --- |

No. ER/ES

Audit Trail Log

| Input reason when saving file | If an existing (already saved) data files and measurement parameters files are edited, a window to input the reason for editing is displayed when saving the file. |
| --- | --- |
| Output audit trail to application log | Enable the audit trail log recorded when the data files and measurement parameters files were saved to be displayed in the application log of the [Log Browser] window as well. |
| Request to reenter the password when a reason is entered | The password is required to enter the change reason for the audit trail. |

1 ON 2 ON

3 - 2 LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]

1.1 System Administration Policy Settings

| Name | Function |
| --- | --- |
| Select a reason for the audit trail from the selection items | Select the reason from the preset selection item list when entering the change reason for the audit trail. Set the selection items in (Selection Administration) of [Administration Tools] on the [LabSolutions Main] window. For details of the "Selection Administration", refer to LabSolutions Help. |

No. ER/ES 4 -

1

Report

| Output data status in header of report | Output the data file's information to a report when printing without saving the data file. |
| --- | --- |
| Prohibit printing of unsaved files | Prohibits printing of files being edited. |

5 ON

6 ON

Others

| Prohibit saving data file as other name | Prohibits duplication of data files by disabling [Save as...] in the [File] menu. |
| --- | --- |
| Prohibit deleting of measured data | Prohibits deletion of measured data in the sample table. |
| Prohibit re-editing of measured data | Prohibits re-editing of measured data in the sample table. |

7 - 8 ON 9 ON

1.1.2 Audit Trail

If [Input reason when saving file] of the instrument policy is enabled (checked), inputting the reason for changes is required when saving the edited data files and measurement parameters files.

For a data file, the input reason for change is recorded in the data history of the data file.

Change history of the measurement parameter files is also saved in the file and you can see it from the instrument control window.

If [Output audit trail to application log] of the instrument policy is enabled, the changes and reasons can also be viewed in the [Log Browser].

This section explains about the audit trail log of a data file.

For how to view the audit trail log of a measurement parameter file, see "3.3 Checking

the Change History of Measurement Parameters" P.17.

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] 3

1 System Administration

■ Review Audit Trail Log Using the Log Browse

1 Double-click the (Log Browse) icon in [Administration] of the [LabSolutions Main] window.

2 Set filtering conditions and click [Start].

4 LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]

3 Double-click the audit trail row. The [Audit Trail Log] window is displayed.

1.1 System Administration Policy Settings 1

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] 5

1 System Administration

1.2 Numerical Rounding and Number of Displayed Digits

For details, refer to "1.2.8 Numerical Rounding and Number of Displayed Digits" in the LabSolutions Manager CS or LabSolutions Manager DB System Users Guide.

The software's [DataManager] rounding method and number of displayed digits can be batch-set in the [Data Processing Settings] sub-window.

Since system settings are changed in this sub-window, log in with a user ID

having [Change System Settings] rights.

1 Double-click the (System Settings) icon in [Administration] of the [LabSolutions Main] window.

2 Click [Data Proc. Settings].

6 LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]

1.2 Numerical Rounding and Number of Displayed Digits

3 Select the rounding method and number of displayed digits for specific calculation results, and click [OK].

1

If [Default] or [Exponential] is selected as the [Display Type], enter the number

of decimal places in the [Decimal Digits] column, and if [Significant Digits] is

selected, enter the number of digits in the [Significant Digits] column.

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] 7

1 System Administration

1.3 System Operation

For details, refer to "1.3.2 Screen Lock to Protect System" in the LabSolutions Manager CS or LabSolutions Manager DB System Users Guide.

1.3.1 Screen Lock to Protect System

The windows of this software can be locked to protect the system from accidents or manipulation by other operators if the logged in operator temporarily leaves the PC. When the lock function is enabled, the window is minimized and displayed as an icon on the taskbar. If the icon is clicked, the [User Authentication] window opens to prompt the user to enter the user ID and password.

This section describes an example of how to lock screens in the Spectrum measurement window.

The procedure for locking the TOC analyzer operation window is described here as an example.

1 Click [Screen Lock] in the [Tools] menu.

The postrun window is minimized and an icon is displayed on the taskbar.

2 Click the [Sample Table Editor] icon on the taskbar to open the application. 8 LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]

3 Enter a [User ID] and [Password], and click [OK]. Only the user who locks the window can release the lock.

1.3 System Operation

1

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] 9

2 Data Management This chapter explains the data administration functions that are specific to theTOC model. For details on other functions, refer to the LabSolutions Manager CS or LabSolutions DB System Users Guide.

2.1 Data Management Using DataManager

[DataManager] allows for database information to be viewed for managing each file used by LabSolutions such as data files, method files, batch files, and report format files, and allows for the narrowing down of necessary data.

In addition, management is carried out while maintaining the links among the files. Therefore, it is easy to check the history and file version.

LabSolutions TOC files registered in [DataManager] are sorted according to the following types:

• Data

• Method

2.1.1 LabSolutions TOC File Formats The following types of LabSolutions TOC files can be managed by [DataManager].

■ Data

Ext.

dpll

Name Description

This is the measurement result data file.

Data profile

The peak area value data, concentration data, measurement parameter information, file information, and data history are included.

10 LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]

2.1 Data Management Using DataManager

Ext.

tlxl

Name Description This is the results table file used for measurement.

Sample table file

The batch table information, measurement parameters, calibration curve parameters, file information, and data history are included.

The data profile consists of the raw TOC data. The sample table file is a batch table file for performing measurement, though it can also be used to view the measurement result information.

2

The file version of sample table file is updated for each sample masurement. So, it may be difficult to view the data profile when [History] is turned on in the DataManager during file version management.

In this case, specify "dpll" in [Data File Name] of [filtering] and filter. If a communication failure occurs with the server when using the client server version, the application ends normally after the current sample measurement is completed.

In this case, the data profile of the sample that was being measured when the failure occurred is automatically registered in the database after communication with the server is restored.

However, it may not be possible for the sample table file to save the sample information that was being measured when the failure occurred.

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] 11

2 Data Management

■ Method

Ext.

insl

cals

cald

metl

tpll

tscl

Name Description

System settings file

Calibration curve settings file

Calibration curve data file

The instrument settings, option settings, and change history are saved in this file.

The concentration range, measurement parameters, and change history are saved in this file.

The concentration range, calibration curve, measurement parameters, and change history are saved in this file.

Method file The measurement parameters, calibration curve to use, and change history are saved in this file.

Control sample file

The measurement parameters, calibration curve to use, results check criteria, and change history are saved in this file.

Schedule file This is the template file for the sample table file. 12 LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]

2.2 Other Functions in DataManager 2.2 Other Functions in DataManager

2.2.1 Importing Local Files in the PC to the Database

For details, refer to "2.2.1 Import Files on the PC to the Database" in the LabSolutions

Manager CS or LabSolutions Manager DB System Users Guide.

With the LabSolutions [DataManager], there is a function for registering measurement 2

parameters files and report files saved in local PC folders to the database.

It is possible to import data files, but data information cannot be extracted using

the DataManager import function. Register data files to the database after

loading them to LabSolutions TOC using the import function of LabSolutions TOC

postrun application ([Import] from the [File] menu).

2.2.2 Data Set and Result File Operations

For details, refer to "2.2 Other Functions in DataManager" and "■ Import Files Related to

Data Acquisition Results as Result Files" in "2.2.3 Data Set and Result File Operations" in

the LabSolutions Manager CS or LabSolutions Manager DB System Users Guide.

■ Import Files Related to Data Acquisition Results (Data Files) as Result Files

Files can be registered to the database related to a row (i.e. sample information) displayed in [Sample Information View] only when [Data] is selected as the file type.

• To import a file, log in by a user ID having the [Import Files] rights.

• [DataManager] has a function for directly referencing the content of backup

databases. Files, however, cannot be imported to backup database since

backups are read-only.

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] 13

2 Data Management

2.2.3 Relating the File Type of Registered Data to Programs

For details, refer to "2.2 Other Functions in DataManager" and "2.2.5 Relate the File Type of Registered Data to Programs" in the LabSolutions Manger CS or LabSolutions Manager DB System Users Guide.

With [DataManager], files contained in rows of sample information can be related to programs used for viewing and printing.

As the default, the following relations are set.

Ext.

dpll, dpl

tlxl

cals

cald

metl

tpll

Name Description

Data profile TocDataProfile.exe

Sample table file TOCSplTbl.exe

Calibration curve settings file TOCSplTbl.exe

Calibration curve data file TOCSplTbl.exe

Method file TOCSplTbl.exe

Control sample file TOCSplTbl.exe

14 LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]

3 History Management

This chapter explains the operations for checking the change history of sample tables and measurement parameters created in LabSolutions TOC.

3.1 Creating History LabSolutions TOC has the following functions for creating history.

Name Description

Change history of sample table

Revision history of

measurement parameters

Change history of system settings file

This is the history that records all changes made to the sample table, from the time it was created until now.

3

This is the history that records all changes made to the measurement parameter file, such as changes to the method file, from the time it was created until now.

This is the history that records all changes made to the system settings file for connecting to the instrument, from the time it was created until now.

The measurement results data creation history and the measurement start/stop operation history are recorded in the application log of the Log Browser.

For details on the method for confirming history information from Log Browse, refer to "1.4.1 View History Information from the Log Browser" in the "LabSolutions Manager CS" or "LabSolutions Manager DB" System Users Guides.

If [Input reason when saving file] of the instrument policy is enabled (checked), inputting the reason for changes is required when saving the edited data files and measurement parameters files.

For how to input a reason for change, see "1.1.2 Audit Trail" P.3.

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] 15

3 History Management

3.2 Checking the Change History of a Sample Table

View the change history that is saved in the sample table file by using the LabSolutions TOC measurement application.

1 Click [View] - [History].

The history of the current sample table is displayed.

16 LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]

3.3 Checking the Change History of Measurement Parameters

3.3 Checking the Change History of Measurement Parameters 3.3.1 Change History of a Measurement Parameter File in the Measurement Application

View the change history of a measurement parameter file by using the LabSolutions TOC measurement application.

This section describes an example of how to view the data history of the method file.

1 Open the measurement parameter file.

3

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] 17

3 History Management

2 Click the [History] tab.

When clicking a desired history line (with the date, time and user name when the change is made) in the upper pain, the content of the change is displayed in the middle pane and the reason for the change in the lower pane.

A reason for change can be input only when [Input reason when saving file] of the system administration policy is enabled.

For the system administration policy, see "1.1 System Administration Policy Settings" P.1.

18 LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]

3.3 Checking the Change History of Measurement Parameters

3.3.2 Change History of a Measurement Parameter File in a Project

View the change history of a measurement parameter file in a project by using DataManager.

1 Double-click the (DataManager) icon in [Administration] of the [LabSolutions Main] window.

2 In DataManager, select "Method" as the type of files.

3

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] 19

3 History Management

3 Right-click a desired measurement parameter line and click [Open with Related Application] from the menu.

The details and change history of the selected measurement parameter are displayed.

20 LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]

3.4 Checking the Change History of the System Settings File

3.4 Checking the Change History of the System Settings File

View the change history of the system settings file by using the LabSolutions TOC measurement application.

1 Click [Opetion] - [Hardware List].

3

The [H/W Settings List] window opens.

2 Click [TOC01].

The [Instrument Propaties] window opens.

LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume] 21

3 History Management

3 Click the [History] tab.

When you click a row in the history displayed in the upper box (the date and user name are shown for each change), the details of the change are shown in the middle box, and the reason for the change is shown in the lower box.

22 LabSolutions™ DB/CS System Users Guide [LabSolutions TOC Volume]